Final Project

BMU Verification Plan

# Context

- List Of Abbreviations

- Introduction

- Design Overview

- Verification Strategy

- Testbench structure

- Verification Extra Details

# List Of Abbreviations

| Abbreviation | Definition |
| --- | --- |
| BMU | Bit Manipulation Unit |
| UVM | Universal Verification Methodology |
| DUT | Design Under Test |
| RTL | Register Transfer Level |
| OP | Operation |
| RISC-V | Reduced Instruction Set Computer Version 5 |
| Zbb | Basic Bit Manipulation Subset |
| Zba | Address Calculation Bit Manipulation Subset |
| Zbs | Single Bit Manipulation Subset |
| Zbp | Permutation Bit Manipulation Subset |
| SRL | Shift Right Logical |
| SRA | Shift Right Arithmetic |
| MSB | Most Significant Bit |
| CSR | Control Status Register |

# Introduction

Goals (Purpose) of this document

Scope of verification

Assumptions and constraints

Constraints:

• No Constrains.

Assumptions:

- The design and verification environment uses UVM .

- Functional coverage goals are achievable and measurable.

- All external dependencies, such as clocks and resets, are provided correctly.

- The RTL design is synthesizable.

# Design Overview

The BMU is a synthesizable RTL block that implements bit manipulation functionality compliant with the RISC-V Bit Manipulation extension. The unit supports instructions from Zbb, Zbs, Zbp, and Zba subsets and handles bit-level arithmetic, logical, and data packing operations. It is integrated into a larger processor pipeline and contributes to the integer execution path.

Key features

- Complement

- with RISC-V Bit manipulation extensions as follow:

- Configurable instruction enablement using compile-time parameters .

- Robust error detection logic: The BMU includes mechanisms to detect and respond to invalid operations or unexpected conditions or even wrong outputs.

- Clock-gated PC register for low-power optimization.

Block diagram and interface

The interface and signals details are listed below in the table:

Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1 bit | input | Clock signal to the module |
| rst_l | 1 bit | input | Active-Low synchronous reset , reset all internal registers of the design to its initial state |
| a_in | 32 bits | input | Represents the first operand of BMU module |
| b_in | 32 bits | input | Represents the second operand of BMU module |
| scan_mode | 1 bit | input | Scan test mode control |
| valid_in | 1 bit | input | Instruction valid flag , indicate if the input instruction is valid or not |
| ap | Struct | input | Decoded instruction control signals (supported instructions by the BMU) |
| csr_ren_in | 1 bit | input | CSR read-enable |
| csr_rddata_in | 32 bits | input | CSR read data |
| result_ff | 32 bits | output | Final computed result |
| error | 1 bit | output | Error Indicator |

# Verification Strategy

## Methodology

UVM will be used to

## Selection of Verification Tests

Tests will cover all aspects of the BMU functionalities, including:

- logical  Operations

This section defines logical operations performed by the BMU, primarily controlled by the ap.op field.The operation mode depends on the ap.Zbb extension bit to be asserted and requires all other control fields to be disserted.

  - Standard Bitwise OR Operation

  - Standard Bitwise Invertor OR Operation

  - Standard Bitwise XOR Operation

  - Standard Bitwise Invertor XOR Operation

- Shifting and Masking Operation

The Shifting and Masking operations include logical shifts, arithmetic shifts, rotates, and bit-level operations. Each operation is enabled by a dedicated control signal in the ap register. Only one shifting/masking operation should be active at a time, and all other ap.* fields must be 0, unless explicitly required by the operation.

  - Right Logical Shift Operation (SRL)

  - Right Arithmetic Shift Operation (SRA)

  - Rotate Right Operation ( ROR )

  - Bit Inverse Operation (BINV)

  - Shift Left by 2 and Add (SH2ADD)

- Arithmetic Operations

- Subtraction (a – b )

- Bit Manipulation Operations

- Set on Less Than (Unsigned SLT)

- Set on Less Than (SLT signed == default)

- Count Trailing Zero Bits (CTZ)

- Count Set Bits / Population Count (CPOP)

- Sign Extend Byte (siext_b)

- Maximum (max)

- Pack (pack)

- Byte-Reverse Register (grev)

- CSR operations

- Unsupported Opcode Handling

- Random cases

# Testbench structure

## Tests

For each operation i will implement a test which contains all the pre-mentioned sequences

## Coverage targets

- Code (line) coverage

- Functional coverage

- Toggle coverage

- Path coverage

- Cross coverage ( advanced , check multiple options once and for testing direct sequence .. )

# Verification Extra Details

Components : driver , monitor , sequencer , sequence_item , agent , environment ,scoreboard , subscriber .

Interfaces: middle-ware interface between the DUT and the top testbench module

Tools and technologies:

- MobaxTerm Server .

- Intergrated Metrics Center (IMC) .

- Cadence Verisium for debugging .

- Visual Studio Code .

Regression Tests:

- Create Regression test to test all cases i have set up and run them together instead of running them individually in the top testbench module .

Coverage Reports: will be attached later

Verification Results: will be attached later